单轮实验方案生成 Benchmark 测试题

| 适用对象 | 不同大模型与智能体框架的实验方案生成能力评测 |
| --- | --- |
| 题库范围 | 电催化、光催化、热催化及水系电池电极材料、共32道单轮题 |
| 评分方式 | 结构化硬校验 + 六个内容维度评分，总分30分 |

编写原则：题干采用正常科研任务叙事，不显式标注“研究对象、研究体系、研究目标”等字段，让模型自行解析任务信息；但每道题均自然包含材料对象、实验场景、研究目的和输出要求。

一、统一作答要求

以下32道题均为单轮实验方案生成任务。被测模型或智能体应根据题干内容，自行理解实验对象、实验目标、对照需求和关键流程，并基于平台提供的 skill 生成可执行 JSON。

题目不再逐项列出平台可用功能，被测对象需要根据 skill 自行选择合适工作站、operation 和参数。

电极材料相关任务仅基于电化学工作站可支持的测试方式组织方案，重点包括 CV、EIS、不同扫描速率下的电化学行为比较，以及基于 CV 曲线的电荷或容量估算。

除题目额外说明外，不应假设未在 skill 中定义的平台能力。

二、题库结构

| 模块 | 方向 | 题数 | 题号 | 主要能力考察 |
| --- | --- | --- | --- | --- |
| A | OER电催化 | 2 | A01-A04 | 催化剂制备、OER测试、对照设计 |
| B | HER电催化 | 2 | B01-B04 | HER催化剂制备、前驱体与热处理比较 |
| C | UOR电催化 | 2 | C01-C02 | 尿素氧化体系与OER/UOR差异比较 |
| D | EOR电催化 | 2 | D01-D02 | 乙醇氧化体系与催化剂对比 |
| E | 水系电池电极材料 | 8 | E01-E08 | 材料制备、CV/EIS评价、结构-电化学关联 |
| F | 光催化体系 | 8 | F01-F08 | 光催化材料制备、反应条件与活性来源验证 |
| G | 选择性催化加氢-热催化 | 8 | G01-G08 | 催化剂合成及筛选，选择性加氢反应机理与催化性能优化 |

三、单轮测试题

A01 Effect of Fe Regulation in NiFe LDH Catalysts on Alkaline OER Performance

To investigate the effect of Fe incorporation on the alkaline OER activity and reaction kinetics of Ni-based hydroxides, please design a reasonable experimental plan based on NiFe LDH catalysts. The experiment should include rational material design and appropriate control samples, enabling comparison between NiFe LDH and Fe-free or different-Fe-content samples in terms of structure, valence state, and OER performance. The plan should cover catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

A02 Comparative Study of Activity-Origin Differences between CoOx and NiFe LDH in Alkaline OER

To compare the activity, reaction kinetics, and stability differences between Co-based oxides and NiFe LDH in alkaline OER, please design a reasonable comparative experimental plan based on CoOx and NiFe LDH catalysts. The experiment should include comparable material design and control sample settings, enabling comparison of the two types of catalysts in terms of structure, composition, surface valence state, interfacial features, and OER performance. The plan should cover catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

B01 Effect of Mo Incorporation in NiMo-Based Catalysts on Alkaline HER Performance

To investigate the effect of Mo incorporation on the alkaline HER activity and reaction kinetics of Ni-based catalysts, please design a reasonable experimental plan based on NiMo-based catalysts. The experiment should include rational material design and appropriate control samples, enabling comparison between NiMo catalysts and Mo-free or different-Mo-content samples in terms of structure, composition, interfacial features, and HER performance. The plan should cover catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

B02 Effect of Different Precursor Routes on the Structure and Performance of CoMo-Based HER Catalysts

To investigate the effect of different precursor routes on the structural formation, element distribution, and alkaline HER performance of CoMo-based catalysts, please design a reasonable parallel control experimental plan based on CoMo-based catalysts. The experiment should include rational precursor-route design and appropriate control samples, enabling comparison of CoMo catalysts obtained from different routes in terms of crystal phase, morphology, composition, surface valence state, and HER performance. The plan should cover catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

C01 Experimental Design for Improving UOR Yield Using Ni-Based Catalysts

To improve the urea conversion rate and target product yield of Ni-based catalysts in alkaline urea oxidation reaction, please design a reasonable experimental plan based on Ni-based catalysts. The experiment should include rational material design, reaction condition optimization, and control test settings, enabling comparison of UOR electrochemical behavior, urea consumption, and liquid-phase product generation under different catalyst compositions, reaction potentials, or reaction times. The plan should cover key steps including catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, electrochemical workstation testing, and liquid-phase product detection of electrolytes before and after reaction. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

C02 Comparative Study of Reaction Behavior Differences of NiCo Catalysts under OER and UOR Conditions

To investigate the differences in activity, reaction kinetics, and interfacial reaction behavior of the same NiCo-based catalyst under alkaline OER and UOR conditions, please design a reasonable control experimental plan based on NiCo-based catalysts. The experiment should include unified material synthesis and electrode preparation conditions, enabling comparison of the catalyst in alkaline electrolytes with and without urea in terms of electrochemical performance, surface valence-state changes, and bubble behavior. The plan should cover catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

D01 Experimental Design for Improving EOR Yield, Selectivity, and Catalyst Lifetime Using Co-Based Catalysts

To improve ethanol conversion, target product selectivity, and catalyst lifetime of Co-based catalysts in alkaline ethanol oxidation reaction, please design a reasonable experimental plan based on Co-based catalysts. The experiment should include rational material design, reaction condition optimization, and control test settings, enabling comparison of EOR electrochemical behavior, ethanol consumption, major liquid-phase oxidation product generation, and stability changes under different catalyst compositions, reaction potentials, or reaction times. The plan should cover key steps including catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, electrochemical workstation testing, long-term stability evaluation, and liquid-phase product detection of electrolytes before and after reaction. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

D02 Comparative Study of Active-Site Differences between Ni-Based and Co-Based Catalysts in Alkaline EOR

To compare the activity, stability, and product conversion differences between Ni-based and Co-based catalysts in alkaline EOR, please design a reasonable parallel control experimental plan based on Ni-based and Co-based catalysts. The experiment should include comparable material design, electrode preparation, and testing conditions, enabling comparison of the two types of catalysts in terms of structure, composition, surface valence state, electrochemical performance, and liquid-phase product distribution. The plan should cover catalyst synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, electrochemical workstation testing, and liquid-phase product detection. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E01 Investigation of Ion Storage Capability of PBA Cathode Materials in Aqueous Potassium-Ion Systems

To investigate the redox behavior and interfacial charge-transfer characteristics of PBA cathode materials in aqueous potassium-ion systems, please design a reasonable experimental plan based on PBA cathode materials. The experiment should include rational material synthesis and electrode construction methods, enabling the acquisition of CV testing results, redox peak positions, peak current changes, and interfacial impedance characteristics during K⁺ insertion/extraction. The plan should cover key steps including PBA material synthesis, necessary post-treatment, structural characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E02 Investigation of Reversible Zn Storage Capability of MnO₂ Cathode Materials in Aqueous Zinc-Ion Systems

To investigate the redox reversibility and interfacial reaction characteristics of MnO₂ inorganic electrode materials in aqueous zinc-ion systems, please design a reasonable experimental plan based on MnO₂ cathode materials. The experiment should include rational material synthesis and control test settings, enabling the acquisition of CV testing results, redox peak changes, peak separation changes, and EIS characteristics of MnO₂ during Zn²⁺-involved reaction processes. The plan should cover key steps including material synthesis, necessary post-treatment, structural characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E03 Effect of Multi-Metal Composition Regulation on the Aqueous Electrochemical Behavior of PBA Cathode Materials

To investigate the effect of multi-metal composition regulation on the aqueous ion-storage behavior, redox process, and interfacial charge transfer of PBA cathode materials, please design a reasonable comparative experimental plan based on multi-metal PBA cathode materials. The experiment should include reasonable control settings using undoped or low-component PBA samples and multi-metal PBA samples, enabling comparison of different samples in terms of structure, composition, valence state, CV testing results, redox peak positions, peak currents, and EIS characteristics. The plan should cover key steps including material synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E04 Effect of Post-Treatment on the Structural Stability and Electrochemical Reversibility of V-Based Oxide Electrodes

To investigate the effect of post-treatment processes on the structural stability and electrochemical reversibility of V-based oxide electrode materials in aqueous systems, please design a reasonable comparative experimental plan based on V-based oxide electrode materials. The experiment should include reasonable control settings under different post-treatment conditions, enabling comparison of the materials in terms of crystal structure, morphology, surface state, peak shape changes during repeated CV scans, redox peak shifts, and EIS characteristics. The plan should cover key steps including material synthesis, post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E05 Study on Improving the Structural Stability of V-Based Cathode Materials in Aqueous Zinc-Ion Batteries

To improve the structural stability and electrochemical cycling reversibility of V-based cathode materials in aqueous zinc-ion batteries, please design a reasonable experimental plan based on V-based cathode materials. The experiment should include rational material design, structural regulation, and control sample settings, enabling comparison of V-based cathode materials under different modification strategies or treatment conditions in terms of crystal structure, morphology, composition, interfacial features, CV testing results, redox peak changes, and EIS characteristics. The plan should cover key steps including material synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E06 Effect of Defects and Crystal Water on Ion-Transport Kinetics of PBA Cathode Materials

To investigate the effect of defect structures or crystal water content on the aqueous ion-transport kinetics and redox reversibility of PBA cathode materials, please design a reasonable comparative experimental plan based on PBA cathode materials. The experiment should include reasonable control settings using PBA samples obtained under different synthesis or post-treatment conditions, enabling comparison of the samples in terms of crystal structure, defect degree, crystal-water-related features, CV peak current changes, peak position shifts, and EIS characteristics. The plan should cover key steps including PBA material synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E07 Comparison of Electrochemical Reversibility of Different Inorganic Anode Materials in Aqueous Systems

To compare the redox reversibility, interfacial impedance, and cycling stability of different inorganic anode materials in aqueous electrolytes, please design a reasonable control experimental plan based on two inorganic anode materials. The experiment should include comparable material synthesis, electrode preparation, and testing conditions, enabling comparison of the two types of anode materials in terms of peak shape retention during repeated CV scans, redox peak separation changes, peak current decay, and EIS characteristics. The plan should cover key steps including material synthesis, necessary post-treatment, structural characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

E08 Effect of Different Synthesis Processes on the Structure and Electrochemical Behavior of Aqueous Electrode Materials

To investigate the effect of different synthesis processes on the structural formation, interfacial characteristics, and electrochemical behavior of aqueous electrode materials, please design a reasonable comparative experimental plan based on one inorganic electrode material. The experiment should include reasonable control settings using samples obtained through different synthesis routes, enabling comparison of the samples in terms of crystal structure, morphology, composition distribution, surface valence state, CV testing results, redox peak changes, and EIS characteristics. The plan should cover key steps including material synthesis, necessary post-treatment, structural/interfacial characterization, electrode preparation, and electrochemical workstation testing. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F01: Preparation and Basic Evaluation of Metal Oxide-Based Photocatalysts for the Hydrogen-Evolution Half-Reaction:

To obtain the photocatalytic performance of metal oxide-based semiconductor photocatalysts in the water-reduction hydrogen-evolution half-reaction under an air atmosphere, design a complete experimental protocol. The protocol should cover material synthesis, necessary thermal treatment, cocatalyst loading, selection of the oxidation half-reaction, and hydrogen detection. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F02: Reaction Evaluation for Photocatalytic Oxidative Coupling of Amines to Imines with Concomitant H2O2 Production

To evaluate the ability of carbon nitride-based composite semiconductor photocatalysts to promote the oxidative coupling of organic amines to imine products under an air atmosphere, design an experimental protocol. The experiment may investigate material-tuning strategies such as calcination temperature, alkali-metal doping, or photodeposited cocatalysts, and compare their effects on substrate conversion, imine-product selectivity, and H2O2 formation/accumulation behavior. The protocol should cover catalyst preparation, necessary post-treatment, photocatalytic reaction testing, analysis of organic products and H2O2, H2O2 decomposition controls, and necessary validation experiments. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F03: Photocatalytic Selective Oxidation of Ethanol to 1,1-Diethoxyethane:

To investigate the performance of metal oxide-based semiconductor photocatalysts for the selective oxidation of ethanol to 1,1-diethoxyethane under an air atmosphere, design an experimental protocol. The experiment may explore material-tuning strategies such as different metal species, metal-composition numbers, synthesis temperatures, heteroatom doping, or photodeposited cocatalysts, and compare their effects on ethanol conversion, 1,1-diethoxyethane selectivity, and the distribution of intermediates such as acetaldehyde. The protocol should cover catalyst preparation, necessary post-treatment, photocatalytic reaction testing, product analysis, and control/validation experiments. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F04: Verification of the Origin of Enhanced Activity in Carbon Nitride-Based Photocatalytic Amine Oxidative Coupling with H2O2 Generation:

After modification, carbon nitride-based composite semiconductor photocatalysts exhibit improved activity for benzylamine oxidative coupling to imines with simultaneous H2O2 generation under an air atmosphere. Design a set of control experiments to distinguish whether the enhanced activity is more likely to arise from carbon nitride structural modulation, the role of the cocatalyst, H2O2 generation/decomposition behavior, or changes in the dominant reactive species. The experiment should establish a complete workflow from control-sample preparation and photocatalytic reaction testing to product analysis and mechanistic validation. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F05: Selective Photocatalytic Oxidation of Organic Sulfides over Bismuth Oxyhalide-Based Photocatalysts:

To investigate the performance of bismuth oxyhalide-based two-dimensional photocatalysts for the selective oxidation of organic sulfides to sulfoxides under visible-light irradiation, design an experimental protocol. The experiment may examine halogen composition, exposed crystal facets, oxygen-vacancy concentration, or noble-metal single-atom loading, and compare their effects on sulfide conversion, sulfoxide selectivity, overoxidation by-product formation, and catalyst recyclability. The protocol should cover catalyst preparation, structural and surface-defect characterization, photocatalytic selective oxidation testing, quantitative product analysis, reactive oxygen species trapping, necessary control validation. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F06: Evaluation of MOF-Derived Photocatalysts for C–N Coupling Reactions:

To evaluate the ability of MOF-derived porous metal oxides or metal/carbon composite photocatalysts to promote C–N coupling between aromatic amines and aldehydes to form imines under mild conditions, design an experimental protocol. The experiment may investigate MOF precursor type, pyrolysis atmosphere, metal-node composition, and pore-structure differences, and compare their effects on substrate conversion, imine selectivity, light dependence, and catalyst structural stability. The protocol should cover precursor synthesis, derivatization treatment, pore-structure and metal-valence characterization, photocatalytic C–N coupling testing, product analysis, dark-reaction and catalyst-free controls. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F07: Photocatalytic Hydrodehalogenation of Haloarenes over Perovskite Quantum Dots:

To investigate the performance of inorganic perovskite quantum-dot photocatalysts for reductive dehalogenation of haloarenes under an inert atmosphere, design an experimental protocol. The experiment may examine halide composition, surface ligands, quantum-dot size, and electron-donor type, and compare their effects on dehalogenation conversion, product selectivity, quantum-dot photostability, and side-reaction formation. The protocol should cover quantum-dot synthesis, surface-ligand treatment, optical-property and stability characterization, photocatalytic dehalogenation testing, product analysis, radical-intermediate trapping, necessary blank controls. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

F08: Photocatalytic Oxidative Hydroxylation of Arylboronic Acids over Covalent Organic Frameworks:

To evaluate the ability of covalent organic framework photocatalysts to catalyze the oxidative hydroxylation of arylboronic acids to phenolic compounds in the presence of oxygen, design an experimental protocol. The experiment may investigate COF linkage type, donor-acceptor unit combination, crystallinity, and pore-environment modulation, and compare their effects on arylboronic acid conversion, phenol selectivity, substrate scope, and photocatalytic cycling stability. The protocol should cover COF synthesis, structural and light-absorption characterization, photocatalytic oxidative hydroxylation testing, quantitative product analysis, oxygen-source verification, reactive oxygen species identification, substrate-scope evaluation. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G01 Selective Hydrogenation of Multifunctional Compounds

To achieve selective hydrogenation of multifunctional compounds, design a comprehensive experimental workflow for a metal oxide–metal composite catalyst system. The study should include the synthesis and optimization of the composite catalysts, characterization, and catalytic hydrogenation reactions under optimized conditions. Evaluate the catalytic activity, chemoselectivity, and cyclic stability of the catalysts toward substrates containing different functional groups through catalytic performance and recycling experiments, and establish structure–performance relationships between the catalyst architecture and hydrogenation selectivity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G02 Optimization of Catalytic Activity for Selective Hydrogenation of Aromatic Hydrocarbons

To enhance the catalytic activity for the selective hydrogenation of aromatic hydrocarbons, design a comprehensive experimental workflow for a nickel-based catalyst system. The study should include catalyst synthesis and characterization, optimization of selective hydrogenation reaction conditions, and detailed product distribution analysis. Systematically investigate the effects of reaction temperature, hydrogen pressure, and reaction time on catalytic activity and hydrogenation selectivity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G03 Application of Hydrogenases in Asymmetric Catalytic Hydrogenation

To explore the application of hydrogenases in asymmetric catalytic hydrogenation, design a comprehensive experimental workflow for a hydrogenase-based catalytic system. The study should include the selection and screening of suitable hydrogenases, optimization of hydrogenation conditions for asymmetric substrates such as prochiral olefins, and systematic evaluation of catalytic activity, reaction selectivity, and stereoselectivity. Investigate the effects of reaction parameters, including temperature and substrate concentration, on catalytic performance and enantioselectivity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G04 Effect of Acid Sites on Catalytic Hydrogenation Performance

To investigate the influence of acidic sites on catalytic hydrogenation performance, design a comprehensive experimental workflow for an acidic catalyst system. The study should include catalyst preparation, systematic evaluation of hydrogenation reactions over catalysts with different acid site concentrations, and investigation of the regulatory effects of acidic centers on catalytic activity and reaction selectivity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G05 Application of Multistep Tandem Catalytic Hydrogenation Reactions

To optimize the multistep tandem catalytic hydrogenation of dimethyl terephthalate, design a comprehensive experimental workflow for a composite catalyst system. The study should include catalyst synthesis and characterization, optimization of reaction conditions, and systematic investigation of the selectivity and conversion toward 1,4-cyclohexanedimethanol. Evaluate the effects of catalyst activity, reaction temperature, and hydrogen pressure on catalytic performance and product distribution. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G06 Synergistic Selective Hydrogenation Using Homogeneous-Heterogeneous Catalysts

To investigate the role of homogeneous–heterogeneous catalysts in synergistic selective hydrogenation reactions, design a comprehensive experimental workflow for a hybrid homogeneous–heterogeneous catalytic system. The study should include catalyst synthesis and optimization of catalytic reaction conditions, with systematic investigation of the synergistic effects between different catalysts on catalytic selectivity and activity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G07 Catalytic Conversion of CO2 to Formic Acid via Hydrogenation

To achieve efficient catalytic hydrogenation of CO₂ to formic acid, design a comprehensive experimental workflow for a metal catalyst system. The study should include catalyst synthesis and characterization, optimization of CO₂ hydrogenation reaction conditions, and systematic evaluation of formic acid selectivity and CO₂ conversion. Investigate the effects of different metal catalysts on catalytic activity, reaction efficiency, and product selectivity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

G08 Hydrogenation of NAD+ to 1,4-NADH Coupled with Enzymatic Asymmetric Reactions

To identify the optimal reaction system for the hydrogenation of NAD⁺ to 1,4-NADH coupled with enzymatic asymmetric transformations, design a comprehensive experimental workflow for an integrated NAD⁺ hydrogenation–biocatalytic system. The study should include optimization of the NAD⁺ hydrogenation process, evaluation of the generation efficiency and stability of 1,4-NADH, and assessment of the selectivity and yield of the coupled enzymatic asymmetric reactions. Systematically investigate the effects of different catalysts, enzyme types, and reaction conditions on overall catalytic efficiency and asymmetric reaction selectivity. Existing executable experimental templates already available on the platform must not be directly reused for dispatch. Dispatch the experiment to Laboratory 303 and return the ID of the experimental task.

四、统一评分细则

评分采用“结构化输出硬校验 + 内容质量评分”的方式。若输出不是可解析 JSON，或明显不符合预定义执行 JSON 基本结构，可先记录为结构化失败；

4.1 硬指标

| 指标 | 判定方式 | 说明 |
| --- | --- | --- |
| JSON可解析率 | 通过 / 不通过 | 输出是否为可解析的 JSON 结构。 |
| Schema通过率 | 通过 / 不通过 | 字段、类型、嵌套结构和必填项是否符合预定义执行 JSON 要求。 |
| Skill引用有效性 | 记录项 | 是否使用 chemistry-experiment-workstation 中的相关 skill，而不是自行虚构平台能力。 |

4.2 Content Scoring Dimensions

| Dimension | 5-Point Capability Range | 3-Point Capability Range | 1-Point Capability Range |
| --- | --- | --- | --- |
| Constraint Compliance | The output appropriately selects the workstation, operation, and parameters, and complies with skill capability boundaries and key constraints. | The output calls some relevant skills, but has missing parameters, insufficient understanding of boundaries, or localized compliance risks. | The output extensively fabricates or misuses skills, lacks key parameters, and clearly deviates from platform capabilities. |
| Scientific Soundness of the Experimental Plan | The synthesis, processing, electrode preparation, and testing pathways are scientifically valid, and the variable and control designs can answer the research question. | The overall plan is acceptable, but parameter settings are generic, the testing logic is not rigorous enough, or the validation process is weak. | The plan is scientifically untenable, with serious errors in the core experimental workflow, testing method, or material logic. |
| Completeness of the Experimental Plan | The plan covers key stages from sample preparation, post-treatment, electrode or reaction-system preparation through testing and validation, forming a closed workflow. | The main workflow is present, but some key stages or result-validation information are missing. | Only partial steps are provided; major experimental processes are missing, so the output cannot constitute a complete experimental design. |
| Token Consumption and Expression Efficiency | The output has no obvious redundancy; token consumption is relatively low among different model outputs for the same question, without compromising plan completeness. | Token consumption is at a medium level among different model outputs for the same question, and the content remains basically relevant. | Token consumption is at the highest level among different model outputs for the same question. |
| Plan Generation Time and Iteration Efficiency | Total time is short; an experimental plan meeting the requirements can be generated quickly, and the generation process is stable and efficient. | Total time is moderate; the plan can be generated within an acceptable time, but there are some review failures, parameter-validation failures, or repeated adjustments. | Total time is long; plan generation takes clearly too long, with frequent review failures or parameter-validation failures, resulting in very low generation efficiency or even failure to obtain an executable plan. |
| Success Rate of Real Experiment Dispatch | The dispatch success rate is at a relatively high level among all test results. | The dispatch success rate is at a medium level among all test results. | The dispatch success rate is at a relatively low level among all test results. |

4.3 总分与能力区间

每个维度1-5分，四个维度总分20分。推荐能力区间如下：

| 总分区间 | 能力等级 | 解释 |
| --- | --- | --- |
| 26–30 | 高能力区间 | 能稳定生成高质量实验方案，具备较强实际使用潜力。 |
| 21–25 | 较高能力区间 | 能完成大部分任务，但仍需要一定人工修订。 |
| 16–20 | 中等能力区间 | 具备基础方案生成能力，但稳定性和科学深度一般。 |
| 9–15 | 较低能力区间 | 方案质量偏低，错误较多，实际参考价值有限。 |
| 8 及以下 | 很低能力区间 | 基本不具备可靠的实验方案生成能力。 |

4.4 常见错误类型标注

任务理解偏差：未识别材料对象、反应体系、对照目标或需要获得的信息。

Skill误调用：调用不存在的工作站或 operation，或虚构平台能力。

约束违背：忽略 skill 中必填参数、参数范围、容器或流程要求。

科学性不足：合成路线、测试方法、反应体系或变量设计不合理。

流程缺失：缺少后处理、电极制备、测试验证或对照样品。

结果目标不匹配：生成的实验无法回答题目要求的性能、结构或机理问题。